### TO BE ADDED TO THE GENERAL NOTES

FOR ALL

SIGNING & STRIPING IMPROVEMENT PLANS

- All work and materials shall conform to Caltrans Standard Plans and Sections 56, 82, 84, and 85 of the Caltrans Standard Specifications, latest edition, and City of Ontario Special Provisions for Signing and Striping except as indicated hereon.

- The Contractor shall renew or replace any existing traffic striping and/or pavement markings, which during his operations have been either removed or the effectiveness of which has been reduced.

- All signs not relocated shall be installed new and shall be 3M, Diamond VIP grade or equivalent with Series 1160 premium protective overlay film.

- Signs not mounted on street lights shall be mounted using TELESPAR 20F12-2, 12 gauge post(s) with quick punch holes, TELESPAR 25K-12-S-2.4 anchors and TELESPAR TL-220 drive nuts.

- All signs shall be blocked, or blocked and braced in accordance with note #2 on Caltrans Standard Plan RS2. Blocks shall have a 3/8" hole in the center so that they are held in place between the sign and the post by the center mounting bolt.

- In case of accidents involving safety matters covered by Section 6409.1B of the California Labor Code, the Contractor shall immediately notify the State Division of Industrial Safety.