### TO BE ADDED TO THE GENERAL NOTES

FOR ALL

TRAFFIC SIGNAL IMPROVEMENT PLANS

- All work and materials shall conform to Caltrans Standard Plans and Section 86 of the Caltrans Standard Specifications, latest editions, and City of Ontario Special Provisions for Traffic Signals prepared specifically for this work.

- All work related to electric service shall be performed in conformance with the requirements of the Southern California Edison Company. Obtain approval for exact equipment, pole, and detector locations prior to final placement.

- All pull boxes adjacent to mast arm poles, or with four or more conduits entering shall be #6. All others shall be #5 unless otherwise shown.

- Detector loops shall be spliced in series. Physically adjacent detector loops shall be wound with opposite rotation. Rotation reversal may be accomplished at the pull box by reversing the leads.

- At the direction of the Engineer, the Contractor shall deliver and unload any salvage at the Ontario Municipal Service Center, 1425 South Bon View Avenue, or City Well Site No. 9, on the west side of Columbia Avenue north of Deodar Street.

- Prior to the start of construction the contractor shall obtain from the City a set of special provisions prepared specifically for this work.